How to:

1) Connect your PC and VU+ Uno with a serial cable. Must be 2->3, 3->2 + 5->5.

2) Set Com Port to one, you’re going to use (the one in which you’ve got the RS 232

connector plugged in)

3) Your VU+ is turned off, click "Connect"

4) Turn your VU+ and click “Open” and choose the nfi file you need.

Then click on "Set IP"

6) Click "Flash", you will have to wait for a few minutes

…. and then you’ll see a progress bar working.

7) When the process is finished, "Flash" button will be in active mode (black letters) again, so turn your VU+ off and then on. Hopefully everything will work again.